Comparison of FEA and Predictions For Bar with Multiple Axials Loads

Geometry: Length = 60, Height = 4

Material: E =1000,  = 0.3.

Mesh: 60 x 4 Linear Elements.

Loads:

Left end: Ux = 0 and Uy = 0.

Right end: Ux = 0 and Uy = 0.

Fx = -15 at (x,y) = (20,1), (20,2), (20,3) and Fx = -7.5 on (20,0), (20,4); Fy = 0 on all.     Fx = 5 at (x,y) = (50,1), (50,2), (50,3) and Fx = 2.5 on (50,0), (50,4); Fy = 0 on all.

(These nodal forces produce a net force of -60 and 20 at x = 20 and x = 50.)

60

4

10

20

20

60

FEA Results to Extract

- x and Ux at (x,y) = (35,2)

Analyses to Compare with FEA Results

- Enter FEA stress x and FEA displacement Ux at (x,y) = (35,2) into table.

- Solve axial loading problem, and predict stress and displacement at the point (x,y) = (35,2).  Enter values into tables.

Numerical Results

| Stress and displacements at (x,y) = (35,2); Compare FEA results and predictions based on solving this problem by methods of axial loading |  |  |  |
| --- | --- | --- | --- |
| x (FEA) | x (Axial loading) | Ux (FEA) | Ux (Axial loading) |

Give your analysis of the problem using the methods of axial loading on a separate page that you attach.  Show precisely how you find x and Ux at (x,y) = (35,2).