Comparison of FEA and Predictions For a Block in Shear

A block 3 m by 0.3 m (by 1 m into paper) is bonded to two rigid plates.

(Additional diagrams of modeled configuration on final page.)

- Block properties: E = 2x106 (2E6) and  = 0.4.

- Vertical displacement of plates = 0.

- Horizontal displacements on plates are shown in Figure.

- Plates are rigid, so displacements are uniform in each plate.

1 mm

3 mm

3 m

0.3 m

A

B

(a) Analyze with methods from class and textbook.  Assume block has uniform shear strain. G = E/(2(1+)).

- Determine the shear stress in block.

- Determine the external horizontal force that must be applied to each rigid plate.

(b)  Analyze this configuration with the Mini-FEA program.

Geometry: Length = 3, Height = 0.3

Material: E =2E6,  = 0.4.

Mesh: 10 x 4 Linear Elements.

Loads:

Apply Ux = -0.003 and Uy = 0 (from top plate) on all nodes at y = 0.3

Apply Ux = -0.001 and Uy = 0 (from bottom plate) on all nodes at y = 0

Analyses to Compare with FEA Results

(i) Draw Shape

- Draw the original rectangular body and the deformed shape on top of each other.

(ii) Stress Comparison

- Enter FEA stress xy at points A and B (each is at center line of block) into tables.

- Use analysis of part (a) to predict stress xy at points A and B into tables.

(iii) Comparison of forces applied by plates

- Extract Fx at all nodes at y = 0 and sum to get net force.  Do same for forces on nodes at y = 0.3.   Enter net forces into Table.

- Use analysis of part (a) to predict net force applied by each of the plates. Enter into table.

(ii) Stress Comparison

| Stress xy at points A and B. Compare FEA results and predictions based on simple shearing. |  |  |
| --- | --- | --- |
|  | A | B |
| xy (FEA) |  |  |
| xy (Simple Shear) |  |  |

Give your simple shear analysis of the block and two rigid plates below.

Then, list the points (A and/or B) at which the predictions of the stress xy agree reasonably well.  Discuss why the agreement is good at these points and not at others points.

(iii) Comparison of forces applied by plates

| Net force on plates. Compare FEA results and predictions based on simple shearing. |  |  |
| --- | --- | --- |
|  | Bottom plate | Top plate |
| Net Fx (FEA) |  |  |
| Net Fx (Simple Shear) |  |  |

Show here how the simple shear analysis leads to your prediction of the forces on the plates.

The FEA analysis in this assignment models the following problem.

1 mm

3 mm

Initial configuration of block and plates

Configuration of block and plates after top and bottom plates have been displaced to the left by different distances