Comparison of FEA and Predictions For Two Bonded Blocks in Shear

Two blocks, each 2 m by 0.2 m (by 1 m into paper), are bonded to three rigid plates.

- Block properties: E = 2.8x109 (2.8E9) and  = 0.4.

- Vertical displacement of plates = 0.

- Horizontal displacements on plates are shown in Figure.

- Plates are rigid, so displacements are uniform in each plate.

2 m

1 mm

3 mm

2 mm

0.2 m

0.2 m

A

B

C

D

(a) Analyze with methods from class and textbook.  Assume blocks have uniform shear strain. G = E/(2(1+)).

- Determine the shear stress in each block.

- Determine the external horizontal force that must be applied to each rigid plate.

(b)  Analyze this configuration with the Mini-FEA program as a single block.

Geometry: Length = 2, Height = 0.4

Material: E =2.8E9,  = 0.4.

Mesh: 10 x 4 Linear Elements.

Loads:

Apply Ux = 0.001 and Uy = 0 (from top plate) on all nodes at y = 0.4

Apply Ux = -0.002 and Uy = 0 (from middle plate) on all nodes at y = 0.2

Apply Ux = 0.003 and Uy = 0 (from bottom plate) on all nodes at y = 0

Analyses to Compare with FEA Results

(i) Stress Comparison

- Enter FEA stress xy at points A, B, C, D (each is at center line of a block) into tables.

- Use analysis of part (a) to predict stress xy at points A, B, C, D into tables.

(ii) Comparison of forces applied by plates

- Extract Fx at all nodes at y = 0 and sum to get net force.  Do same for forces on nodes at y = 0.2 and y = 0.4.   Enter net forces into Table.

- Use analysis of part (a) to predict net force applied by each of the three plates. Enter into table.

(i) Stress Comparison

| Stress xy at points A, B, C, and D. Compare FEA results and predictions based on simple shearing. |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | A | B | C | D |
| xy (FEA) |  |  |  |  |
| xy (Simple Shear) |  |  |  |  |

Give your simple shear analysis of the two blocks and three rigid plates below.

Then, list the points (A,B,C,D) at which the predictions of the stress xy agree reasonably well.  Discuss why the agreement is good at these points and not at others points.

(ii) Comparison of forces applied by plates

| Net force on three plates. Compare FEA results and predictions based on simple shearing. |  |  |  |
| --- | --- | --- | --- |
|  | Bottom plate | Middle plate | Top plate |
| Net Fx (FEA) |  |  |  |
| Net Fx (Simple Shear) |  |  |  |

Show here how the simple shear analysis leads to your prediction of the forces on the plates.