Comparison of FEA and Predictions of Bending Stress and Curvature

Geometry: Length = 20, Height = 4     (Diagram of modeled configuration on final page)

Material: E =30.0E6,  = 0.3.

Mesh: 20 x 4 Quadratic Elements.

Loads:

Left end: All nodes at left end x = 0 (not just those shown in Figure) do not displace horizontally or vertically (Ux = 0, Uy = 0).

Right end:  Fx = 1000 at (x,y) = (20,0);  Fx = -1000 at (x,y) = (20,4)

20

4

x

y

1,000

1,000

FEA Results to Extract

- x at (x,y) = (10,0), (10,1), (10,2), (10,3), and (10,4).

- Uy at (x,y) = (9,2), (10,2), and (11,2).

Analyses and Comparison with FEA Results

(i) Stress Comparison

- Enter FEA stress x at points (10,0), (10,1), (10,2), (10,3), and (10,4) into tables.

- Use simple bending to predict stresses at these points, and enter into tables.  Remember how y is defined in the bending stress formula  = -My/I.

(ii) Curvature Comparison

- Enter FEA deflections at points (9,2), (10,2), and (11,2) into Table.

- Estimate the slope at x = 9.5 using [Uy(10,2) - Uy(9,2)]/(10 – 9).  Enter into table.

- Estimate the slope at x = 10.5 using [Uy(11,2) - Uy(10,2)]/(11 – 10). Enter into table.

- Estimate the curvature at x = 10 using [Slope(10.5,2) - Slope(9.5,2)]/(10.5-9.5).  Enter into Table.

- Predict the curvature from M/EI and enter into Table.

Results

(i) Stress Comparison

|  | (10,0) | (10,1) | (10,2) | (10,3) | (10,4) |
| --- | --- | --- | --- | --- | --- |
| x (FEA) |  |  |  |  |  |
| x (-My/I) |  |  |  |  |  |

Show the individual terms for evaluating x = -My/I

(i) Curvature Comparison

|  | (9,2) | (10,2) | (11,2) |
| --- | --- | --- | --- |
| Uy |  |  |  |

|  | (9.5,2) | (10.5,2) |
| --- | --- | --- |
| Slope (Uy/x) |  |  |

|  |  = (Slope)/x |  = M/EI |
| --- | --- | --- |
| At x = 10 |  |  |

Show the individual terms for evaluating  = M/EI

The FEA analysis in this assignment models the following problem.

Relatively Rigid Supporting Wall

Applied

Moment

Body is attached to the wall which prevents any displacement of the body.

Body